### hear file from a PC with .img

#### ! ATTENTION !

- If the TV-Box works well for what we need not update, unless we know we're doing NOT UPDATE

- There are many TV-Box especially with cheap copies called "clones" that are incompatible with the official roms of the original models, it is impossible to know whether or not we have a clone without opening the device and buy it with another original. We recommend buying devices in stores that recommend and if only known models is possible.

- To update using the flash tool that comes with the ROM , if it is not any can be used in this article. 
Older versions are incompatible and give errors during the process.

- You must always use a connector USB 2.0

- Process for Windows 8 and Windows 10  if you give us problems detection: 
Before running the Setup program flasheo need to disable the secure signing of drivers. 
- Windows 8 : in environment Metro> Update and retrieve> Recovery> Advanced Home> restart now> Troubleshooting> boot Options>  Disable the mandatory use of signed drivers 
- Windows 10:   Start Menu> Settings> Update and security> Recovery> 
Under the option advanced Home click restart now> Troubleshooting> Options advanced> startup Settings> Reset> Press 7 or F7 for D eshabilitar the mandatory use of signed drivers.

- NOTE: The computer will restart and you can install drivers without digital signatures. If you restart the computer again the mandatory use of signed drivers will be enabled.

#### Installation Process

1- Download and install the tool AMLogic on your PC from  HERE  (read previous notes for Windows 8/10).During installation will install the drivers concerned. If we see the menus in Chinese have the option of putting English in the second top menu.

2 Restart your PC

3- Run the tool USB_Burning_Tool is installed on your PC and from the tab File / Import image select the file.Img you have downloaded the firmware to install.

4- Now with your TV-Box off connect it to the USB-MicroUSB OTG cable from the PC port. Now press the button recovery  device (for Minix U1 just have to leave pressed the power button until you exit the robot) and simultaneously turn on the TV-Box. If all went well the flash tool will recognize your TV-Box as in the image.

- NEVER tick box "Reset after success" and "Overwrite key"

- If we have a message in red MAC = 0, read section 1.1 of this manual

5. Click Start and expects the percentage counter reaches 100%. Once completed the process starts again the TV-Box, will take longer than usual to start, it is best that you leave untouched a few minutes after starting and restarting again.

#### 1.1 Correct Error MAC = 0

If we have this message is that we lack the license files for TV-Box, with the rom or utility manufacturer or shop must provide us a file with different files inside, we have to copy those files within the folder " license " where we installed the utility  USB Burning Tool .